STP Series Installation Instructions

Prescription Tillage Technology

LLC. 103 S Broadway Street

Red Oak, Iowa 51566

712-357-1149

Prescriptiontillage.com

Safety

A brief description of signal words that may be used in this installation brochure:

| Note | Used to address safety practices not related to personal injury. |
| --- | --- |
| Caution | Indicates a potentially hazardous situation that, if not avoided, may result in minor or moderate injury. It may also be used to alert against unsafe practices. |
| Warning | Indicates a potentially hazardous situation that, if not avoided, could result in serious injury or death. It may also be used to alert against unsafe practices. |
| Danger | Indicates an imminently hazardous situation that, if not avoided, will result in serious injury or death. This signal word is to be limited to the most extreme situations. |

|  | Danger |
| --- | --- |
|  | Consult your implement and tractor operators manual for correct and safe locking position of the unit in the upward or raised position. FAILURE MAY RESULT IN PERSONAL INJURY OR DEATH. |

|  | Warning |
| --- | --- |
|  | Servicing machine in the upward or raised position; position locking devices in proper position and or install safety stand to prevent accidental drop or fall and injury. |

|  | Warning |
| --- | --- |
|  | When handling blades wear protective foot- wear such as steal toed boots. No open toed shoes. |
|  | Warning |
|  | Only use hub bosses to rotate STP True- V blades. Do not place fingers between blades during rotation. |
|  | Warning |
|  | Blades are sharp, wear leather gloves when handling and installing. |
|  | Danger |
|  | Do not ride on top of the planter unit, serious injury or death will occur. |

CNH-GREAT PLAINS-CrustBuster

Off-Set Shank Configuration Applications

WARNING: Blades are sharp use leather gloves and use caution.

- Use 1 ¾ “x 5/8 “R or L hand threaded bolts for installation of double roller bearing hub assembly. In bolt applications.

- Install the STPF as a Lead Blade Only [The STPF blade is a right or left Lead Blade on each unit]. [The STPF is a lead blade Only!].  (Figure 1) The STPF was designed specifically for 16-40 CEC Soils only in No-Till - Green Cover Planting to Maximize Cut and Soil Flow. Not applicable in Rocks, Shale, Hard pack Soils, Low CEC Soils or Abrasive Soils]

Caution; USE the STPS non-inside counter beveled blade as a lead blade in Extreme Rock, Gravel or Shale, High Speed, pre-plant tillage or Shallow Planting application].  (Figure 2)

Note: 2020 Blade Production are laser etched for direction and part number

Note: 2020 STPS Blade Production is countered beveled, only requiring a left and a right part number.

- Install; The STPS Inside Counter beveled Blade as a Tailing Blade. [The STPS blade is a right or left Trailing blade on each unit]. (Figure 3 & 4)

FIGURE 1

FIGURE 2

FIGURE 3

- Maintain equal inside blade spacing from the Shank, Boot, or Furrow Firming point. (Figure 5 & 6)

- Blade gap spec = 1/16”-1/8". Low CEC soils or preplant tillage recommended spacing of 1/16". High CEC Soils or No-Till-Green Cover planting recommended spacing of 1/8". Use Hub to rotate blades. (Figure 7)

Note: Blades will make contact during soil engagement.

Blade wear: As disc blades wear spacing will need to be adjusted to maintain gap spec.

FIGURE 4	                                       FIGURE 5	                                  FIGURE 6	                                   FIGURE 7

- Figure 3 & 7: Depth wheels should make blade to depth wheel contact at the 4:00-5:00 position, preventing soil build up between blade and depth wheel, lessening row plugging. Reinstall scrapers and gauge wheels assemblies. Raise Gage wheel assembly 1.5"-2" [simulating planting depth]. Push Gauge wheel assembly Inward toward True-V blade, Rotate Depth wheel in a forward direction. Depth wheel should make light or constant contact with True-V Blade